# COA Assignment

Name:

- Abhijith Nambiar – BL.SC.U4CSE24001

- Satya Sreepada - BL.SC.U4CSE24012

# 1. Architectures Used in Computer Systems

## 1.1 x86 Architecture

Type of Architecture: Von Neumann Architecture
Reason: x86 systems use a unified memory space where both instructions and data share the same memory and bus. This simplifies design but can lead to the Von Neumann bottleneck.

Register Set:

- Total General Purpose Registers: 16 (in x86-64)

- Register Size: 64-bit each

- Common Registers:

  - RAX (Accumulator)

  - RBX (Base)

  - RCX (Counter)

  - RDX (Data)

  - RSI, RDI (Index registers)

  - RBP (Base Pointer)

  - RSP (Stack Pointer)

- Special Registers:

  - RIP (Instruction Pointer)

  - RFLAGS (Status/Flags Register)

- SIMD Registers:

  - XMM (128-bit), YMM (256-bit), ZMM (512-bit in advanced CPUs)

- Register Nature:

  - General purpose registers are same size (64-bit)

  - SIMD registers are larger and used for parallel processing

Pipelining:

- Modern x86 processors use deep pipelines (10–20+ stages)

- Typical conceptual stages:

  - Instruction Fetch

  - Decode

  - Micro-operation translation

  - Dispatch

  - Execute

  - Memory Access

  - Write Back

- Hazards:

  - Data Hazards: handled using forwarding and out-of-order execution

  - Control Hazards: branch prediction used extensively

  - Structural Hazards: minimized using multiple execution units

## 1.2 MIPS Architecture

Type of Architecture: Modified Harvard Architecture
Reason: MIPS uses separate instruction and data paths to improve performance, though memory may be unified physically.

Register Set:

- Total Registers: 32 General Purpose Registers

- Register Size: 32-bit (MIPS32)

- Register Naming:

  - $zero (always 0)

  - $t0–$t9 (temporary)

  - $s0–$s7 (saved)

  - $a0–$a3 (arguments)

  - $v0–$v1 (return values)

- Special Registers:

  - Program Counter (PC)

  - HI and LO registers (used in multiplication/division)

- Register Nature:

  - All registers are uniform (same size)

  - Simpler design compared to x86

Pipelining:

- Classic 5-stage pipeline:

  - IF (Instruction Fetch)

  - ID (Instruction Decode)

  - EX (Execute)

  - MEM (Memory Access)

  - WB (Write Back)

- Hazards:

  - Data Hazards: solved using forwarding and stalling

  - Control Hazards: handled using branch delay slots

  - Structural Hazards: avoided by design (separate paths)

# 2. Architectures Used in Mobile / Energy-Constrained Devices

## 2.1 AVR Architecture

Type of Architecture: Harvard Architecture
Reason: AVR strictly separates program memory and data memory, allowing faster access and reduced power consumption—ideal for embedded systems.

Register Set:

- Total Registers: 32 General Purpose Registers (R0–R31)

- Register Size: 8-bit

- Key Features:

  - Registers are directly connected to ALU → faster execution

  - Some registers can form 16-bit pairs (e.g., R26–R31 for pointer operations)

- Special Registers:

  - Program Counter (PC)

  - Stack Pointer (SP)

  - Status Register (SREG)

- Register Nature:

  - All registers are uniform (8-bit)

  - Highly optimized for low-power operations

Pipelining:

- 2-stage pipeline:

  - Instruction Fetch

  - Execute

- Advantages:

  - Simple pipeline reduces power consumption

- Hazards:

  - Minimal data hazards due to simple architecture

  - Control hazards are very limited

  - Structural hazards avoided due to Harvard design

## 2.2 RISC-V Architecture

Type of Architecture: Modified Harvard Architecture
Reason: Designed for efficiency, scalability, and modularity. Uses separate instruction and data paths in most implementations.

Register Set:

- Total Registers: 32 General Purpose Registers

- Register Size:

  - 32-bit (RV32)

  - 64-bit (RV64)

- Register Naming:

  - x0 (always zero)

  - x1 (return address)

  - x2 (stack pointer)

  - x10–x17 (arguments)

- Special Registers:

  - Program Counter (PC)

- Register Nature:

  - All registers are uniform in size

  - Clean and simple design (RISC philosophy)

Pipelining:

- Typically uses 5-stage pipeline (similar to MIPS):

  - IF

  - ID

  - EX

  - MEM

  - WB

- Hazards:

  - Data Hazards: solved via forwarding and pipeline stalls

  - Control Hazards: branch prediction or stalling

  - Structural Hazards: minimal due to simple design

# Conclusion

- x86 is complex and powerful, designed for high-performance computing systems with deep pipelines and advanced hazard handling.

- MIPS represents a clean RISC architecture with simple pipeline design and uniform registers.

- AVR is optimized for low power and embedded systems, making it ideal for energy-constrained devices.

- RISC-V is a modern, flexible architecture gaining popularity due to its open standard and efficient design.

These architectures highlight the trade-offs between complexity, power consumption, and performance in modern computing systems.