# 1. Shift Operators (VHDL)

Shift operators are used in VHDL to manipulate bit values by shifting or rotating them. There are six shift operators:

| Operator | Full Name | Description |
| --- | --- | --- |
| sll | Shift Left Logical | Shifts bits to the left, filling vacated bits with 0s |
| srl | Shift Right Logical | Shifts bits to the right, filling vacated bits with 0s |
| sla | Shift Left Arithmetic | Shifts bits left; the rightmost bit is replicated |
| sra | Shift Right Arithmetic | Shifts bits right; the leftmost (sign) bit is replicated |
| rol | Rotate Left Logical | Rotates bits to the left (bits wrap around from left to right) |
| ror | Rotate Right Logical | Rotates bits to the right (bits wrap around from right to left) |

# 2. Arithmetic Operators (VHDL)

VHDL supports standard arithmetic operators for mathematical computations:

| Operator | Symbol | Operation | Example |
| --- | --- | --- | --- |
| Addition | + | Adds two operands | a + b |
| Subtraction | - | Subtracts the second operand from the first | a - b |
| Multiplication | * | Multiplies two operands | a * b |
| Division | / | Divides the first operand by the second | a / b |
| Modulo | % | Calculates the remainder after division | a % b |
| Exponentiation | ** | Raises the first operand to the power of the second | a ** b |

# 3. VHDL — Advantages & Disadvantages

## 3.1 Advantages of VHDL

- VHDL is a standard language with simpler syntax similar to Ada language.

- It describes the behavior of digital circuits and systems using testbench.

- It is a strongly typed language and supports multi-level abstraction.

- Both signed and unsigned numbers can be used in VHDL.

- VHDL supports readily available tools and is compatible with all CAD tools.

## 3.2 Disadvantages of VHDL

- It has more wordy and complicated coding style.

- Every bit vector use of the keyword 'downto' is very tedious.

- Sometimes a very long sensitivity list must be provided in each process.

- Missing even a single signal in the sensitivity list may cause simulation and synthesis to fail.

- Synthesized hardware from a VHDL description may not always be immediately obvious.

# 4. Physical Constraints in Digital Systems

Physical constraints govern real-world performance limits of digital circuits. The key physical constraints are:

## 4.1 Gate Delay

Gate delay is the time it takes for a logic gate to propagate an input signal to the output. It represents the time difference between when a signal is applied to the input and when the corresponding output change is observed.

- Measured in nanoseconds (ns)

- Affects the maximum operating frequency of the circuit

- Accumulates along the critical path — longer paths = more delay

## 4.2 Fan-In

Fan-in is the maximum number of inputs that can be connected to a logic gate.

- The diagram shows a gate with Fan-in = 4 (4 input lines feeding into one AND gate)

- Higher fan-in increases gate delay due to increased capacitance

- Practical logic gates usually have limited fan-in (typically 2–8 inputs)

## 4.3 Fan-Out

Fan-out is the maximum number of output connections (load gates) that can be driven from the output of a single logic gate.

- The diagram shows Fanout = 4 (one output driving 4 NOT gates)

- Exceeding the fan-out limit can distort output signals and cause logic errors

- A buffer can be inserted to increase effective fan-out

## 4.4 Power

Power consumption is a critical physical constraint especially in embedded systems. It includes static power (leakage) and dynamic power (switching activity). Managing power is essential for battery-operated and heat-sensitive designs.

# 5. Instruction Set Architecture (ISA)

## 5.1 What is ISA?

Instruction Set Architecture (ISA) is the set of machine-level instructions that a CPU can understand and execute. It acts as the interface between hardware and software.

- Written in machine language (binary — 1s and 0s)

- Controls how bits and bytes move within the processor

- Specifies what the processor can do AND how it gets done

- Visible to assembly language programmers, compiler writers, and application developers — essentially a programmer's manual

## 5.2 What ISA Defines

- Supported data types

- Register set and their usage

- How the hardware manages main memory

- Key features such as virtual memory

- Which instructions a microprocessor can execute

- The input/output model

- ISA can be extended by adding new instructions or supporting larger addresses/data values

## 5.3 Examples of ISA

| ISA | Type | Common Usage |
| --- | --- | --- |
| ARM (Advanced RISC Machine) | RISC | Mobile devices — iPhones, iPads, most Android phones |
| MIPS (Microprocessor without Interlocked Pipeline Stages) | RISC | Embedded systems, video game consoles |
| IA-32 | CISC | Dominant ISA for desktops and laptops (x86) |
| PowerPC | RISC | Nintendo Wii, GameCube, network switches and routers |
| Freescale Power Architecture | RISC | Xbox 360, PlayStation 3 |
| Atmel AVR | RISC | Embedded systems like motor cars |

# 6. RISC vs. CISC Processor

RISC (Reduced Instruction Set Computer) and CISC (Complex Instruction Set Computer) are two fundamental CPU design philosophies.

| S.No. | Feature | RISC | CISC |
| --- | --- | --- | --- |
| 1 | Instruction Set | Reduced (smaller set) | Complex (larger set) |
| 2 | Number of Instructions | Less compared to CISC | More compared to RISC |
| 3 | Addressing Modes | Fewer addressing modes | More addressing modes |
| 4 | Instruction Format | Fixed instruction format | Variable instruction format |
| 5 | Power Consumption | Consumes low power | Consumes high power |
| 6 | Pipelining | Highly pipelined | Less pipelined |
| 7 | Optimization | Focuses on software optimization | Focuses on hardware optimization |
| 8 | RAM Requirement | Requires more RAM | Requires less RAM |

# 7. ARM Processor

## 7.1 Introduction

ARM (Advanced RISC Machine) is a popular low-power embedded processor whose core is based on the RISC architecture.

- First prototype introduced in 1985 under the name Acorn RISC Machine, later renamed Advanced RISC Machine (ARM)

- Being a RISC machine, the design is simple — can be built with fewer transistors

- High code density and low power consumption make it ideal for embedded applications

- ARM processor accounts for more than 75% of 32-bit processors in market share (as of 2009)

## 7.2 ARM Load-Store Architecture

ARM is based on the load-store architecture, meaning memory is accessed only via two special instructions:

- LOAD instruction — copies data from memory to the register file

- STORE instruction — writes data from registers back to memory

- All arithmetic and logical instructions access only the register file (not memory directly)

- This keeps operand access time-efficient and simple

- Core started as 32-bit processor, extended to 64-bit from ARM9 family onwards

## 7.3 Evolution of ARM

ARM has evolved through multiple architecture versions, becoming progressively smaller (in nm), more powerful, and more feature-rich:

| Architecture | Processor | Process Node | Key Features | Era |
| --- | --- | --- | --- | --- |
| ARMv4 | ARM7TDMI | 180nm | Thumb instruction set | ~1995 |
| ARMv5 | ARM926EJ | 130nm | VFPv2, Jazelle, Thumb | Late 1990s |
| ARMv6 | ARM1176 | 90nm | Thumb-2, TrustZone, SIMD, VFPv2 | ~2001–2005 |
| ARMv7 A/R | Cortex A9 | 40–65nm | Virtualization, NEON, Thumb-2, TrustZone, SIMD | ~2004–2010 |
| ARMv8 A | Cortex A50 | 14–28nm | Scalar FP, Advanced SIMD, Backward Compatibility, 64-bit | ~2011–2015+ |

# 8. Features of ARM Processor (ARM7TDMI)

## 8.1 Core Specifications

- 32-bit MCU and ALU

- 32-bit Data Bus with aligned memory space (executes 32 bits per machine cycle)

- All instructions are 32 bits wide

- 32-bit Address Bus (can interface up to 4GB memory directly)

- Originally followed Von Neumann Model (4GB memory shared between code and data); newer versions switched to Harvard Architecture

## 8.2 Pipeline & Registers

- Three-stage pipelining: Fetch → Decode → Execute

- 37 registers total, each 32 bits wide; 16 registers visible at a time (R0–R15)

- LOAD-STORE Architecture (separate instructions for LOAD and STORE operations)

## 8.3 Operating Modes & Interrupts

- 7 operating modes

- 7 interrupt/exception types

- 7 addressing modes

## 8.4 Data Formats

- 8 bits = Byte

- 16 bits = Half Word

- 32 bits = Word

## 8.5 ARM7TDMI Acronym Explained

| Letter | Stands For | Description |
| --- | --- | --- |
| T | Thumb Instructions | 16-bit compressed instruction set for code density |
| D | Hardware Debugging | On-chip debug support |
| M | Long Multiply | Enhanced 32x32 = 64-bit multiply instruction |
| I | ICE Microcells | Breakpoints and watchpoints support for debugging |

# 9. Functional Blocks of ARM

## 9.1 Data Flow Overview

The ARM core's functional blocks are interconnected by data buses. Referring to Figure 1.1 (Data Flow Diagram of ARM):

- Data comes in and is passed to the Instruction Decoder and the Sign Extend unit

- Sign Extend unit converts 8-bit/16-bit data to 32-bit before writing to the Register File

- Register File (R0–R15) holds operands and results

- Operands Rn and Rm flow into the ALU (via Barrel Shifter for pre-processing)

- ALU and MAC (Multiply-Accumulate) store results back into the Register File

- Address Register holds the base address generated by the ALU for load/store

- Incrementer auto-increments to access the next sequential memory location

## 9.2 Key Functional Units

### ALU (Arithmetic Logic Unit)

The ALU decodes the arithmetic/logic operation encoded in the instruction and performs it on the bus data and accumulator contents.

- Decodes operational activities of arithmetic and logic in electronic code

- Has three additional control signals tracking 8 specific components

- Includes an Accumulator matching the width of the system bus

- A single-bit 'zero' output goes high when all bits in the accumulator are zero

### MAC (Multiply-Accumulate Unit)

The MAC retrieves values from data paths A and B, multiplies and accumulates, and stores results back in the register file. It works in tandem with the ALU.

### Barrel Shifter

The Barrel Shifter is an integral part of the ARM core used to pre-process the source operand Rm before it reaches the ALU.

- Shifts or rotates a value by a specified number of bit positions in a single clock cycle

- Has a control input specifying how many bit positions to shift

- Implemented with a pattern of shift multiplexers

- 16-bit Thumb instructions have limited shift options; 32-bit ARM instructions give the barrel shifter nearly full exposure

- Can perform complex operations in a single instruction when combined with ALU

### Control Unit

The Control Unit is the main element of the CPU that coordinates all activities during program execution.

- Receives and decodes instructions from memory

- Part of the Von Neumann architecture (first described by John von Neumann)

- Directs memory, ALU, and I/O based on instructions sent to the CPU

- Fetches instructions from main memory into the CPU instruction register

- Creates reference voltage to oversee instruction execution based on register content

- Responsible for context switching — saves/restores CPSR using SPSR

# 10. Booth Multiplier

## 10.1 Multiplier Element

- The multiplier element has three 32-bit inputs returned from the register file

- The output is 32 least significant bits

- Multiplication begins every time the input is asserted (go signal)

- A 'done' signal goes high at the completion edge

## 10.2 Booth Algorithm

The Booth Algorithm is a notable multiplication algorithm for 2's complement numbers.

- Treats both positive and negative numbers equally

- Runs of 0s or 1s in the multiplier are skipped (no addition or subtraction done), enabling faster multiplication

- Multiplication completes within 16 clock cycles

# 11. Data Size and Instruction Set

## 11.1 Register and Data Width

The data processing capability of an instruction depends on the register width. The register file holds signed or unsigned 32/64-bit data depending on the ARM core family.

- ARM sign extend hardware converts Byte (8-bit) and Halfword (16-bit) to Word (32-bit) for register storage

## 11.2 ARM Implements Three Instruction Set Categories

- 32-bit ARM Instruction Set — standard, full-featured

- 16-bit Thumb Instruction Set — compressed for code density

- 8-bit Jazelle Instruction Set — Jazelle cores can execute Java bytecode directly in hardware

The Current Program Status Register (CPSR) decides which instruction set is active. The instruction set typically uses a 3-address format:

- Rd — destination operand

- Rn — first source operand

- Rm — second source operand (may pass through Barrel Shifter)

## 11.3 Instruction Set Feature Comparison

| Feature | ARM | Thumb | Jazelle |
| --- | --- | --- | --- |
| Instruction Size | 32 bit | 16 bit | 8 bit |
| Core Instructions | 58 | 30 | 60% of Java bytecodes in Hardware |
| Register Usage | 15 GPR + PC | 8 GPR + 7 high registers + PC | Hardware-managed |

# 12. ARM Operating Modes (Register Modes)

The ARM processor has 7 basic operating modes — 6 privileged modes and 1 non-privileged (user) mode. Privileged modes are used to service interrupts, exceptions, and access protected resources.

| Mode | Type | Description & Usage |
| --- | --- | --- |
| User Mode | Non-privileged | The mode under which most tasks and application programs run. Limited access to system resources. |
| System Mode | Privileged | Runs user and system programs with full access permissions. Uses the same register set as User Mode. |
| Supervisor Mode | Privileged | Where the OS kernel operates. Entered on reset and when a Software Interrupt (SWI) instruction executes. |
| Fast Interrupt Request (FIQ) | Privileged | Entered when a high-priority (fast) interrupt is raised. Supports channel communication for data transfer. Has banked registers R8–R14 for speed. |
| Interrupt Request (IRQ) | Privileged | Entered when a low-priority (normal) interrupt is raised. Used for general-purpose interrupt handling. |
| Abort Mode | Privileged | Used to handle memory access violations (data aborts and prefetch aborts). |
| Undefined Mode | Privileged | Used to handle undefined instructions not supported by the processor implementation. |

# 13. ARM Registers

## 13.1 General Purpose Registers

The ARM architecture provides sixteen 32-bit general purpose registers (R0–R15):

- R0–R14 — general purpose data storage (15 GPRs)

- R15 — Program Counter (PC); its value changes as the core executes instructions. An explicit write to R15 alters program flow.

- Low Registers: R0–R7 (accessible by all instructions including Thumb)

- High Registers: R8–R12 (accessible primarily by ARM instructions)

- R13 — Stack Pointer (SP)

- R14 — Link Register (LR): stores the return address when a subroutine is called

- R15 — Program Counter (PC)

## 13.2 Register Banking

Depending on which processor mode is active, a register name may map to a different physical storage location. This is called register banking.

- Banked registers use physically distinct storage

- Usually accessible only when executing in that particular mode

- FIQ mode has the most banked registers (R8–R14) to speed up interrupt handling without needing to save/restore registers

- Each privileged mode has its own banked R13 (SP) and R14 (LR)

- All modes share R0–R7

## 13.3 Special Purpose Registers

Registers are broadly classified into:

- Special Purpose Registers (SPR) — processor-driven (CPSR, SPSR)

- General Purpose Registers (GPR) — for the programmer (R0–R12)

Overall there are 37 registers total, of which 16 (R0–R15) are visible to the user at any time:

- Out of 37 registers, 20 are used orthogonally under different banks

- The processor mode determines which bank is accessible

## 13.4 Program Status Registers (PSR)

There are two program status registers:

- CPSR (Current Program Status Register) — reflects the current state of the processor

- SPSR (Saved Program Status Register) — saves a copy of the CPSR before entering a privileged mode

Context Switching Process:

- Current executing mode's CPSR is saved in the SPSR of the initiator mode

- Processor transitions to the new mode

- Task executes in the new mode

- A special return instruction copies the SPSR back into the CPSR, restoring the previous state

# 14. Current Program Status Register (CPSR)

The CPSR is a 32-bit register organized into fields. Referring to Figure 1.3:

| Bit(s) | Field | Description |
| --- | --- | --- |
| 31 | N — Negative flag | Set if the result of an ALU operation is negative |
| 30 | Z — Zero flag | Set if the result of an ALU operation is zero |
| 29 | C — Carry flag | Set if an ALU operation produced a carry out |
| 28 | V — Overflow flag | Set if an ALU operation produced a signed overflow |
| 27 | Q — Sticky Overflow (Saturation) | Architecture 5TE/J only; indicates if saturation has occurred in QADD/QDADD/QSUB/QDSUB or SMLA/SMLAW overflow |
| 24 | J bit | Architecture 5TEJ only; J=1 means processor is in Jazelle state |
| 16–23 | Reserved (x) | Reserved for future use — should not be modified by software |
| 8–15 | Reserved | Reserved/undefined fields |
| 7 | I bit — IRQ disable | I=1 disables the IRQ interrupt |
| 6 | F bit — FIQ disable | F=1 disables the FIQ interrupt |
| 5 | T bit — Thumb state | Architecture xT only; T=0: ARM state, T=1: Thumb state |
| 0–4 | Mode bits | Specify the current processor mode |

## 14.1 Important Notes on CPSR

- Condition Code (CC) flags N, Z, C, V are set based on the outcome of operations

- All Thumb instructions and some ARM instructions affect CC flags

- ARM instructions influence CC only if appended with 'S' suffix (e.g., ADDS, MOVS)

- T/J bits in CPSR should NEVER be changed by directly writing to the PSR — use BX/BLX instructions instead

- The processor is in ARM state on reset, after interrupt, or during exception execution

- Processor mode can be changed using data transfer instructions that write to the mode bits

- A read-modify-write strategy should be used to update status registers to ensure future compatibility

# 15. ARM Instruction Set

## 15.1 Features of ARM Instruction Set

- Load-Store architecture — memory accessed only via LDR/STR

- 3-address instruction format (destination + 2 source operands)

- Conditional execution of every instruction (no wasted cycles on branch mispredictions)

- Possible to load/store multiple registers at once (LDM/STM)

- Possible to combine shift and ALU operations in a single instruction

Instruction Format (Figure 1.2):

[ f bits: function ] [ n bits: op1 addr ] [ n bits: op2 addr ] [ n bits: dest addr ]

Example: ADD d, S1, S2   ;  d := S1 + S2

## 15.2 Instruction Set Categories

ARM instructions fall into three main categories:

- Data Processing — move, arithmetic, logical, comparison, and multiply instructions

- Data Movement — load and store instructions for memory access

- Flow Control — branch and subroutine call instructions

## 15.3 Data Processing Instructions — General Rules

- All operands are 32-bit, coming from registers or immediate literals

- The result (if any) is 32-bit and placed in a register

- Exception: long multiply produces a 64-bit result

- 3-address format: Rd := Rn OP Rm (with optional Barrel Shift on Rm)

- Most data processing instructions can process one operand using the Barrel Shifter

# 16. ARM Data Types

ARM natively supports three data sizes:

| Data Type | Size | Alignment Requirement | Variants |
| --- | --- | --- | --- |
| Byte | 8 bits | Any byte boundary (no alignment needed) | Signed and unsigned |
| Halfword | 16 bits | Must be aligned on 2-byte boundaries | Signed and unsigned |
| Word | 32 bits | Must be aligned on 4-byte boundaries | Signed and unsigned |

# 17. Arithmetic Instructions

These instructions perform mathematical operations on two 32-bit registers and store the result in a destination register.

| Mnemonic | Operation | Description |
| --- | --- | --- |
| ADD r0, r1, r2 | r0 := r1 + r2 | Add two 32-bit values |
| ADC r0, r1, r2 | r0 := r1 + r2 + C | Add two 32-bit values with carry (C flag from CPSR) |
| SUB r0, r1, r2 | r0 := r1 - r2 | Subtract two 32-bit values |
| SBC r0, r1, r2 | r0 := r1 - r2 + C - 1 | Subtract with carry (for multi-word subtraction) |
| RSB r0, r1, r2 | r0 := r2 - r1 | Reverse subtraction (reverses the order of operands) |
| RSC r0, r1, r2 | r0 := r2 - r1 + C - 1 | Reverse Subtract with carry for multi-word subtraction |

## 17.1 How ADC Works (Multi-Word Addition)

ADC is particularly useful when adding numbers larger than 32 bits. The carry flag comes into play when values exceed 32-bit storage.

Example — 64-bit addition using two 32-bit registers:

ADDS R0, R1, R2     ; R0 = R1 + R2 (lower 32 bits), sets carry flag if overflow ADC  R3, R4, R5     ; R3 = R4 + R5 + Carry (upper 32 bits)

- ADDS R0, R1, R2 — adds R1 and R2, stores result in R0; sets the CPSR carry flag (C) if overflow occurs

- ADC R3, R4, R5 — adds R4, R5, and the carry from the previous operation; ensures correct multi-word addition

## 17.2 How RSC Works (Multi-Word Subtraction)

RSC (Reverse Subtract with Carry) is primarily used in multi-word (64-bit) subtraction, ensuring correct borrow handling.

Formula: R0 = R2 - R1 - (1 - C), where:

- R2 is the minuend (number being subtracted from)

- R1 is the subtrahend (number to subtract)

- C is the carry flag from CPSR; (1-C) handles the borrow

Example — 64-bit subtraction:

SUBS R0, R2, R1     ; R0 = R2 - R1, sets carry (C) if no borrow RSC  R3, R4, R5     ; R3 = R5 - R4 - (1 - Carry)

# 18. Logical Instructions

Logical instructions perform bitwise operations on 32-bit register values:

| Mnemonic | Operation | Description |
| --- | --- | --- |
| AND r0, r1, r2 | r0 := r1 & r2 | Bitwise AND of two 32-bit values |
| ORR r0, r1, r2 | r0 := r1 ¦ r2 | Bitwise OR of two 32-bit values |
| EOR r0, r1, r2 | r0 := r1 ^ r2 | Exclusive OR (XOR) of two 32-bit values |
| BIC r0, r1, r2 | r0 := r1 & ~r2 | Logical Bit Clear (AND NOT) — clears bits in r1 that are set in r2 |
| MVN r1, r2 | r1 := ~r2 | Move Bitwise NOT — moves the bitwise complement of r2 into r1 |

## 18.1 Notes on Logical Instructions

- AND is used for masking — isolating specific bits

- ORR is used for setting specific bits

- EOR is used for toggling specific bits

- BIC is used for clearing specific bits (useful for flag manipulation in CPSR)

- MVN is used to load the bitwise complement of a value into a register

# 19. Quick Reference Summary

## ARM Instruction Set — At a Glance

| Category | Instructions | Purpose |
| --- | --- | --- |
| Arithmetic | ADD, ADC, SUB, SBC, RSB, RSC | Math operations on 32-bit values |
| Logical | AND, ORR, EOR, BIC, MVN | Bitwise operations |
| Move | MOV, MVN | Load values into registers |
| Comparison | CMP, CMN, TST, TEQ | Compare and test (affect CC flags only, no result stored) |
| Multiply | MUL, MLA, UMULL, SMULL | 32-bit and 64-bit multiply |
| Load/Store | LDR, STR, LDM, STM | Memory access operations |
| Branch | B, BL, BX, BLX | Flow control and subroutine calls |
| Shift (via Barrel Shifter) | LSL, LSR, ASR, ROR, RRX | Shift/rotate operands before ALU operation |

## ARM Processor — Key Numbers to Remember

| Parameter | Value |
| --- | --- |
| General Purpose Registers | 16 (R0–R15) |
| Total Physical Registers (ARM7) | 37 |
| Register Width | 32 bits |
| Pipeline Stages (ARM7) | 3 (Fetch, Decode, Execute) |
| Operating Modes | 7 (1 non-privileged + 6 privileged) |
| Addressing Modes | 7 |
| Instruction Set Sizes | 32-bit (ARM), 16-bit (Thumb), 8-bit (Jazelle) |
| Core Instructions (ARM) | 58 |
| Core Instructions (Thumb) | 30 |
| Booth Multiplier Completion | 16 clock cycles |