# 1. What is Computer Architecture?

Computer Architecture is the field of study of selecting and interconnecting hardware components to create computers that satisfy functional performance and cost goals.

It refers to those attributes of the computer system that are visible to a programmer and have a direct effect on the execution of a program.

## 1.1 What Does It Cover?

Computer Architecture concerns Machine Organization, interfaces, application, technology, measurement & simulation. It includes:

- Instruction Set — the set of instructions the CPU can execute

- Data Formats — how data is represented in the system

- Principle of Operation — a formal description of every operation

- Features — organization of programmable storage, registers used, interrupt mechanisms, etc.

In Short Computer Architecture = Instruction Set Architecture (ISA) + Machine Organization + Related Hardware

# 2. Evolution of Computers (5 Generations)

## Generation 1 (1940–1950) — Vacuum Tubes

Key Machines ENIAC (1945) and UNIVAC (1950) defined this era.

- Machine: ENIAC (1945)

  - Designed by Mauchly & Echert; built by the US Army

  - Purpose: calculate trajectories for ballistic shells during WWII

  - Used ~18,000 vacuum tubes and 1,500 relays

  - Programmed by manually setting switches (no stored programs!)

- Machine: UNIVAC (1950)

  - First commercial computer ever built

- Concept: John Von Neumann Architecture

  - Goldstine & Von Neumann took ENIAC's idea further

  - Invented the concept of storing a program inside memory

  - This 'stored-program' concept became the foundation for virtually every computer designed since

### Key Features of 1st Generation:

- Used electron-emitting vacuum tube devices

- Data and programs stored in a single read-write memory

- Memory addressable by location (not by content)

- Programmed in Machine Language / Assembly Language

- Executed instructions sequentially (one at a time)

## Generation 2 (1950–1964) — Transistors

William Shockley, John Bardeen, and Walter Brattain invented the transistor, which replaced bulky vacuum tubes with smaller, more reliable switches.

### Key Features:

- Transistors replaced vacuum tubes — smaller size, better reliability

- First Operating Systems appeared (handled one program at a time)

- On-off switches controlled electronically (not mechanically)

- High-level programming languages introduced

- Floating-point arithmetic support added

## Generation 3 (1964–1974) — Integrated Circuits (IC)

Microprocessor chips packed thousands of transistors onto a single computer chip — an entire circuit on one chip.

### Key Features:

- Microprocessor chips with thousands of transistors on one chip

- Semiconductor memory introduced

- Multiple computer models with different performance characteristics

- Computers became drastically smaller in size

## Generation 4 (1971–1980) — VLSI (Very Large Scale Integration)

VLSI circuits containing ~5,000+ transistors and associated circuits on a single chip enabled microcomputers to become practical and affordable.

### Key Features:

- Computers became powerful, compact, reliable, and affordable

- The Personal Computer (PC) revolution began

- Time-sharing, real-time networks, and distributed operating systems were introduced

- High-level languages: C, C++, DBASE, etc.

## Generation 5 (1980–Present) — ULSI & AI

VLSI evolved into ULSI (Ultra Large Scale Integration), producing microprocessor chips with ten million+ electronic components.

### Key Features:

- Based on parallel processing hardware and AI (Artificial Intelligence) software

- AI allows computers to think and reason like humans

- Languages: C, C++, Java, .NET, and more

### Quick Reference: All 5 Generations

| Generation | Period | Technology | Key Feature |
| --- | --- | --- | --- |
| 1st | 1940–1950 | Vacuum Tubes | ENIAC; stored-program concept (Von Neumann) |
| 2nd | 1950–1964 | Transistors | Smaller, reliable; first OS; high-level languages |
| 3rd | 1964–1974 | Integrated Circuits | Thousands of transistors on one chip; miniaturization |
| 4th | 1971–1980 | VLSI | PC revolution; time-sharing; C, C++ |
| 5th | 1980–Now | ULSI + AI | Parallel processing; AI; millions of transistors |

# 3. Components of Computer Architecture

The main components of a computer architecture are the CPU, memory, and peripherals. All these elements are linked by the system bus (address bus + data bus + control bus). There are 8 key components:

## 3.1 Input Unit & Associated Peripherals

- Connects the external environment to the computer

- Receives information from input devices and translates it to machine language

- Inserts translated data into the computer system

- Common devices: keyboard, mouse — each has hardware drivers that allow them to sync with the rest of the architecture

## 3.2 Output Unit & Associated Peripherals

- Delivers the results of computer processing to the user

- Most output data consists of music, graphics, or video

- Output devices: display screen, printer, speakers, headphones, etc.

Example To play an MP3: the system reads a number array from disk into memory → manipulates numbers (decompress audio) → outputs uncompressed audio to the audio chip → chip sends sound through speakers.

## 3.3 Storage Unit / Memory

The storage unit stores data and is split into Primary Storage and Secondary Storage.

### Primary Storage (Main Memory) — CPU has DIRECT access

- Used for storing data and instructions during program execution

- Two types: RAM and ROM

  - RAM (Random Access Memory) — supplies data directly to the CPU; temporary/volatile storage

  - ROM (Read-Only Memory) — contains pre-installed firmware; persistent and cannot be modified; boots the machine and runs POST (Power-On Self Test)

### Secondary Storage — CPU does NOT have direct access

- Data must first be transferred to main memory before CPU can use it

- Permanently stores large amounts of data

- Examples: Hard Disk Drives (HDD), Solid-State Drives (SSD), CDs

## 3.4 Central Processing Unit (CPU)

The CPU is the brain of the computer. It includes registers, an ALU, and control circuits that interpret and execute assembly language instructions.

### CPU Sub-components:

### a) Registers

- High-speed, purpose-built temporary memory devices

- Accessed and modified directly by the CPU during execution

- Store: data, commands, addresses, and intermediate processing results

### b) Arithmetic Logic Unit (ALU)

- Contains electrical circuitry that performs arithmetic and logical operations

- Arithmetic: addition, subtraction, multiplication, division

- Logical: comparisons (<, >, AND, OR, etc.)

- Uses registers to hold data being processed

### c) Control Unit

- Collaborates with input/output devices

- Instructs the computer to execute stored program instructions

- Communicates with the ALU and registers

- Goal: arrange and coordinate data and instruction processing

## 3.5 Buses

A bus is a physical collection of signal lines that carry data between components.

- Buses carry electrical impulses (data) between different hardware components

- Bus size = number of signal lines (e.g., an 8-bit bus carries 8 data bits in parallel)

- Example: USB — Universal Serial Bus

| Bus Type | Purpose |
| --- | --- |
| Address Bus | Carries memory addresses from CPU to memory |
| Data Bus | Transfers actual data between components |
| Control Bus | Carries control signals (read/write commands) |

# 4. Eight Great Ideas in Computer Architecture

These are foundational design principles used by computer architects when designing modern systems:

| # | Idea | What It Means |
| --- | --- | --- |
| 1 | Design for Moore's Law | Anticipate that transistor counts double every ~2 years; design for future hardware, not just today's |
| 2 | Use Abstraction to Simplify Design | Hide complex low-level details behind simpler interfaces so engineers can work at a higher level |
| 3 | Make the Common Case Fast | Optimize the operations that occur most frequently for the best overall speedup |
| 4 | Performance via Parallelism | Do multiple things simultaneously to complete work faster |
| 5 | Performance via Pipelining | Break tasks into stages and overlap their execution (like an assembly line) |
| 6 | Performance via Prediction | Guess what will be needed next and prepare ahead; correct if wrong |
| 7 | Hierarchy of Memories | Use a layered memory system (registers → cache → RAM → disk) balancing speed vs. cost |
| 8 | Dependability via Redundancy | Add redundant components so the system keeps working even if one part fails |

# 5. Types of Computer Architecture

## 5.1 Instruction Set Architecture (ISA)

Definition ISA is the bridge between software and hardware. It is the programmer's view of the machine.

- Computers only understand binary (0s and 1s); humans write in high-level languages

- ISA translates high-level language into binary instructions the CPU can execute

### Types of ISA Instructions:

- Arithmetic/Logic Instructions — perform mathematical or logical operations on one or more operands

- Data Transfer Instructions — move data between memory and processor registers (or vice versa)

- Branch & Jump Instructions — interrupt the normal sequential flow and jump to another instruction address

## 5.2 Microarchitecture

- Focuses on how ISA instructions are actually implemented at the hardware level

- Unlike ISA (which describes what instructions exist), microarchitecture describes how they execute

- Influenced by the microprocessor's structural design (circuit layout, pipelines, caches)

## 5.3 Client-Server Architecture

- Multiple clients (remote processors) send requests to a centralized server (host computer)

- Server receives requests and sends replies; client computers display results to users

- Server provides a standardized, transparent interface — clients don't need to know internal server details

- Clients: desktops/laptops. Servers: powerful hardware on the network

- Most efficient when clients and servers have pre-specified responsibilities

## 5.4 SIMD — Single Instruction, Multiple Data

- Processes multiple data points simultaneously using the same instruction

- Cleared the path for supercomputers and high-performance computing

- All processors receive the same instruction from the control unit but operate on different data packets

- The shared memory unit must interact with all CPUs concurrently

## 5.5 Multicore Architecture

- A single physical processor chip contains the logic of multiple processors (cores)

- Multiple processing cores are integrated onto a single integrated circuit

- Goal: perform more tasks concurrently to improve overall system performance

- Example: Modern CPUs with 4, 8, 16, or more cores

# 6. Classic Architecture Examples

## 6.1 Von Neumann Architecture (Princeton Architecture)

Origin Established in a 1945 paper by John von Neumann: 'First Draft of a Report on the EDVAC'

Key idea: Both program instructions AND data share the same memory and buses (unified bus).

### 5 Proposed Components:

- A processor with connected registers

- A control unit capable of storing instructions

- Memory capable of storing both data and instructions — communicates via buses

- Additional or external storage

- Device input as well as output mechanisms

Limitation Because data and instructions share the same bus, the CPU cannot fetch an instruction and read/write data at the same time — this is called the 'Von Neumann bottleneck'.

## 6.2 Harvard Architecture

The Harvard architecture uses separate (distinct) storage and signal pathways for data and instructions.

- Unlike Von Neumann, data memory and instruction memory are physically separate

- Allows the CPU to fetch an instruction AND access data simultaneously — faster

- In practice, a modified Harvard architecture with two distinct caches (one for data, one for instructions) is commonly used

- Used by: X86 and ARM (Advanced RISC Machine) systems

| Feature | Von Neumann | Harvard |
| --- | --- | --- |
| Memory | Single shared memory | Separate instruction & data memory |
| Buses | Shared instruction + data bus | Separate instruction bus & data bus |
| Speed | Can be slower (bottleneck) | Faster (simultaneous fetch + data access) |
| Complexity | Simpler design | More complex hardware |
| Used In | Most general-purpose computers | DSPs, microcontrollers, ARM, X86 cache |

# 7. Combinational and Sequential Logic Circuits

## 7.1 Combinational Circuits

Definition A combinational circuit produces outputs that depend ONLY on the current inputs — no memory of past inputs.

- No memory elements — the output changes instantly when inputs change

- Outputs are immediate and directly reflect input changes

- Used for tasks like: addition, subtraction, logical AND/OR/NOR operations

### Examples:

- Adders (e.g., half adder, full adder)

- Multiplexers (MUX)

- Encoders and Decoders

### Advantages:

- Simplicity — easier to design without memory elements

- Speed — output adjusts instantly with input changes

- Resource Efficiency — needs fewer components than equivalent sequential circuits

### Disadvantages:

- Limited Functionality — cannot perform operations that need historical data

- Complexity Grows — becomes hard to design when there are many inputs

## 7.2 Sequential Circuits

Definition A sequential circuit produces outputs based on BOTH current inputs AND past states (memory). It uses memory elements like flip-flops or latches.

- Employs memory components (flip-flops, latches, registers)

- Output depends on current inputs + the stored previous state

- Requires a clock signal to synchronize state changes

- More functional and versatile than combinational circuits

### Examples:

- Counters

- Shift Registers

- Flip-Flops

- State Machines (Finite State Machines)

### Advantages:

- Memory Utilization — stores previous states to enable complex operations

- Functional Versatility — handles sequences of operations, counters, state machines

### Disadvantages:

- More Complex — harder to design due to memory elements and clock management

- Slower Operation — output may be delayed due to dependency on previous states and clock cycles

## 7.3 Combinational vs Sequential — Full Comparison

| Aspect | Combinational Circuit | Sequential Circuit |
| --- | --- | --- |
| Output depends on | Current inputs only | Current inputs + past states (memory) |
| Memory Elements | None required | Flip-flops, latches required |
| Clock Signal | Not required | Required to sync state changes |
| Timing | Output is immediate | Depends on clock pulses |
| Design Complexity | Simpler | More complex |
| Speed | Faster | Slower (clock-dependent) |
| Power Consumption | Lower | Higher (memory + clock circuitry) |
| Applications | Arithmetic logic (adders, MUX, encoders) | Counters, shift registers, state machines |

# 8. Hardware Description Languages (HDLs)

Hardware Description Languages (HDLs) are special programming languages used to describe the structure and behavior of electronic circuits.

- Used to design: processors, motherboards, FPGAs (Field Programmable Gate Arrays), ASICs, CPLDs

- Unlike regular programming languages, HDLs work with respect to clock signals and timing delays

- Time is a critical parameter in HDLs — they model how hardware behaves over time

## 8.1 VHDL — Very High Speed Integrated Circuit Hardware Description Language

Full Form VHDL = VHSIC HDL, where VHSIC = Very High Speed Integrated Circuit

VHDL describes the behavior of an integrated circuit or system and is used to implement the physical circuit or system.

### Key Properties of VHDL:

- Standard — follows IEEE standards

- Portable — code can be used across different platforms

- Reusable — write once, use in multiple designs

- Vendor Independent — not tied to any specific hardware vendor

- Multi-level Abstraction — can model digital systems at many levels (behavioral, structural, etc.)

### Where is VHDL code used?

- CPLDs — Complex Programmable Logic Devices

- FPGAs — Field Programmable Gate Arrays

- ASIC chips — Application-Specific Integrated Circuits (fabricated chips)

## 8.2 Verilog

Background Developed by Gateway Design Automation in the mid-1980s, later acquired by Cadence Design Systems.

Verilog is an HDL used to describe digital systems and circuits in the form of code. It is widely used for design and verification of ASICs and FPGAs.

### Key Properties of Verilog:

- Supports a range of abstraction levels: structural → behavioral

- Used for both simulation-based design and synthesis-based design

- Suitable for digital and mixed-signal systems

## 8.3 Why Is Verilog Better Than Older HDLs?

- Simpler Syntax — easier and faster to write; fewer errors compared to VHDL

- Better Behavioral Modeling — supports gate-level to behavioral-level modeling; easier to describe complex circuits

- Higher Level of Abstraction — uses modules and ports for a more efficient design process

## 8.4 Verilog Modeling Styles

Verilog has three modeling styles, each suited to different design situations:

### 1. Data Flow Modeling

- Specifies how data flows through the circuit using concurrent signal assignment statements

- No specified ordering of statements — ordering doesn't matter

- Requires Boolean equations for circuit specifications

- Boolean equations are a series of equations defining signal relationships

- Focuses on HOW data flows through the system, not on individual components

- Suitable for simple logic — expresses data flow efficiently

- Can become complicated for designs with complex control flow

### 2. Behavioral Modeling

- Follows a procedural approach — similar to writing a program

- Uses control flow statements like if-then-else to define how the design reacts to inputs

- The programmer writes a set of instructions that decide the system's behavior under different conditions

- Flexible — great for modeling complex logic with control flow

- Suitable for describing intricate algorithms within hardware

- Drawback: harder to visualize exactly how the hardware is physically built (compared to structural modeling)

### 3. Structural Modeling

- Designer does not need to design the internal working of each component individually

- Simply use pre-existing components by giving them unique names (instances)

- Provides a clear, circuit-like view — each block is a known entity

- Since it uses pre-verified components, troubleshooting (debugging) is easier

- Focuses on connections between components rather than internal behavior

| Modeling Style | Focus | Best For | Drawback |
| --- | --- | --- | --- |
| Data Flow | How data moves through the circuit (Boolean equations) | Simple logic and arithmetic | Complex control flow becomes messy |
| Behavioral | What the system does (procedural, like programming) | Complex algorithms, state machines | Harder to visualize physical hardware |
| Structural | How components connect together (netlist) | Large designs with reusable blocks | Requires pre-designed sub-components |

# 9. VHDL Operators

VHDL uses operators to perform computations and comparisons on signals and variables.

## 9.1 Logical Operators

These operators perform Boolean logic operations on their operands:

| Operator | Symbol | Description | Output True When... |
| --- | --- | --- | --- |
| AND | & | Logical AND operation | BOTH operands are true |
| OR | ¦ | Logical OR operation | At LEAST ONE operand is true |
| NOT | ~ | Logical negation — flips the value | Input is false (flips 0→1, 1→0) |
| XNOR | XNOR | Exclusive NOR — checks equality | BOTH operands are same (both T or both F) |
| NAND | NAND | Logical AND then inverted | Output is FALSE only when BOTH are true |
| NOR | NOR | Logical OR then inverted | Output is TRUE only when BOTH are false |

Memory Tip NAND = NOT AND. Output is 0 only when both inputs are 1. In all other cases output is 1. NOR = NOT OR. Output is 1 only when both inputs are 0.